Software Design Document

GPIO_Driver Module

Version 1.1.0

Authors: J.M. Páez Sandoval · L.A. Lugo Muñiz · D. Lopez Moreno · A. Higuera Lopez

CETYS Universidad — Facultad de Ingeniería

Microcontroladores y Sistemas Embebidos

# 1. Introduction

## 1.1. Purpose

This document describes the design and architecture of the gpio_driver module for Lab 2. It covers the updated implementation that adds full Alternate Function configuration, atomic BSRR-based pin writes, output type and speed control, and a GPIO_Status_t error-return type.

## 1.2. Scope

The gpio_driver is a low-level C module for the STM32F411RE. It uses CMSIS GPIO_TypeDef pointers and provides a clean API for configuring digital I/O, alternate functions (e.g. TIM1_CH1 on PA8), and atomic pin operations. It is used by the led, pwm, and timer modules.

## 1.3. Definitions, Acronyms, and Abbreviations

| Term | Definition |
| --- | --- |
| GPIO | General-Purpose Input/Output peripheral |
| CMSIS | Cortex Microcontroller Software Interface Standard |
| AF | Alternate Function — routes pin to a peripheral (e.g. TIM1_CH1) |
| MODER | Mode Register — 2 bits per pin: 00=Input, 01=Output, 10=AF, 11=Analog |
| OTYPER | Output Type Register — 0=Push-pull, 1=Open-drain |
| OSPEEDR | Output Speed Register — 2 bits per pin (slew rate) |
| PUPDR | Pull-Up/Pull-Down Register — 2 bits per pin |
| AFR | Alternate Function Register — AFRL (pins 0–7), AFRH (pins 8–15) |
| BSRR | Bit Set/Reset Register — atomic set (bits[15:0]) / reset (bits[31:16]) |
| IDR | Input Data Register — read-only pin states |
| ODR | Output Data Register — XOR for toggle |
| GPIO_Status_t | Return type: GPIO_OK=0, GPIO_ERROR=1 |

## 1.4. Requirements

All requirements for this module are allocated in Docs/SRS/GPIO_Driver_SRS.md (provided by the instructor without modification).

# 2. System Architecture

## 2.1. High-Level Design

Application Layer: main.c, led.c, pwm.c — call gpio_driver APIs; no register access.

GPIO_Driver Layer: All GPIO register operations isolated here. Peripheral clock enabled automatically in gpio_init().

Hardware Layer: STM32F411RE GPIO peripheral registers accessed via CMSIS GPIO_TypeDef pointers (GPIOA…GPIOE).

## 2.2. Module Interfaces

Public API: Declared in gpio_driver.h. All functions available to any including module.

Dependencies: stm32f411xe.h (CMSIS GPIO_TypeDef, RCC), stdint.h, stddef.h (NULL). No HAL.

# 3. Detailed Design

## 3.1. Class Diagram

Figure 1. Class Diagram — gpio_driver module, enums, and dependent modules

## 3.2. Module Behavior

### Sequence Diagram 1 — gpio_init() + AF Configuration

Figure 2. Sequence Diagram — Full pin initialization: gpio_init(), gpio_set_output_type(), gpio_set_speed(), gpio_set_af()

### Activity Diagram 1 — gpio_init()

Figure 3. Activity Diagram — gpio_init() with NULL/range validation

### Sequence Diagram 2 — gpio_write() (BSRR) + gpio_toggle()

Figure 4. Sequence Diagram — Atomic gpio_write() via BSRR, gpio_toggle() via ODR XOR

### Activity Diagram 2 — gpio_write()

Figure 5. Activity Diagram — gpio_write() BSRR atomic set/reset

### State Diagram — GPIO Pin Lifecycle

Figure 6. State Diagram — GPIO pin states: UNINITIALIZED → INPUT/OUTPUT/AF/ANALOG

## 3.3. GPIO_Driver Module Description

gpio_init() enables the peripheral clock via the private gpio_enable_clock() helper, validates port and pin, then writes the 2-bit MODER field using a read-modify-write.

gpio_set_output_type(), gpio_set_speed(), and gpio_set_pupd() apply the same read-modify-write pattern to OTYPER (1 bit), OSPEEDR (2 bits), and PUPDR (2 bits) respectively.

gpio_set_af() selects AFRL (AFR[0]) for pins 0–7 or AFRH (AFR[1]) for pins 8–15, writing the 4-bit AF field at the correct bit offset. All functions return GPIO_ERROR if port is NULL or pin > 15.

gpio_write() uses BSRR for atomic operation: writing bit n of BSRR[15:0] sets ODR bit n HIGH; writing bit n of BSRR[31:16] resets it LOW. No read-modify-write is needed, eliminating race conditions.

gpio_toggle() XORs the ODR register directly. This is non-atomic (read-modify-write on ODR) and should only be used in single-threaded contexts.

gpio_read() reads the IDR register and masks the target bit. Returns 1 if HIGH, 0 if LOW.

# 4. Application Programming Interfaces

## 4.1. Data Types and Macros

Enumeration types defined in gpio_driver.h:

| Enum | Values | Notes |
| --- | --- | --- |
| GPIO_Mode_t | INPUT=0, OUTPUT=1, AF=2, ANALOG=3 | MODER 2-bit field value per pin |
| GPIO_OType_t | PP=0, OD=1 | OTYPER 1-bit field per pin |
| GPIO_Speed_t | LOW=0, MEDIUM=1, HIGH=2, VHIGH=3 | OSPEEDR 2-bit field per pin |
| GPIO_PUPD_t | NONE=0, PULLUP=1, PULLDOWN=2 | PUPDR 2-bit field per pin |
| GPIO_AF_t | AF0…AF15 (0–15) | AFR 4-bit field per pin |
| GPIO_Status_t | OK=0, ERROR=1 | Return type for all public functions except void ones |

## 4.2. Error Codes

GPIO_OK (0U) — operation completed successfully. GPIO_ERROR (1U) — invalid argument: port==NULL or pin>15 or AF>15. Void functions (gpio_write, gpio_toggle, gpio_read) have no return code.

## 4.3. Function Definitions

### gpio_init

| Function Name | gpio_init |
| --- | --- |
| Syntax | GPIO_Status_t gpio_init(GPIO_TypeDef *port, uint8_t pin, GPIO_Mode_t mode) |
| Parameters (in) | port — GPIOA…GPIOE pointer pin — 0–15 mode — GPIO_MODE_INPUT / OUTPUT / AF / ANALOG |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | GPIO_Status_t — GPIO_OK or GPIO_ERROR |
| Description | Enables GPIO peripheral clock, validates arguments, then writes MODER[2n+1:2n] = mode. Must be called before any other function on the pin. |
| Constraints | Does not configure OTYPER, OSPEEDR, PUPDR, or AFR — call gpio_set_* functions separately after gpio_init(). |
| Available via | gpio_driver.h |

### gpio_set_output_type

| Function Name | gpio_set_output_type |
| --- | --- |
| Syntax | GPIO_Status_t gpio_set_output_type(GPIO_TypeDef *port, uint8_t pin, GPIO_OType_t otype) |
| Parameters (in) | port — GPIO port pointer pin — 0–15 otype — GPIO_OTYPE_PP (push-pull) or GPIO_OTYPE_OD (open-drain) |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | GPIO_Status_t — GPIO_OK or GPIO_ERROR |
| Description | Configures OTYPER[n] = otype. Push-pull (default) can source and sink current; open-drain only sinks. |
| Constraints | Only meaningful for output-mode pins. Has no effect on input pins. |
| Available via | gpio_driver.h |

### gpio_set_speed

| Function Name | gpio_set_speed |
| --- | --- |
| Syntax | GPIO_Status_t gpio_set_speed(GPIO_TypeDef *port, uint8_t pin, GPIO_Speed_t speed) |
| Parameters (in) | port — GPIO port pointer pin — 0–15 speed — LOW/MEDIUM/HIGH/VHIGH |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | GPIO_Status_t — GPIO_OK or GPIO_ERROR |
| Description | Sets OSPEEDR[2n+1:2n] = speed. Controls output slew rate (rise/fall time). Use GPIO_SPEED_HIGH for PWM output pins. |
| Constraints | Higher speed increases EMI; use the lowest speed sufficient for the application. |
| Available via | gpio_driver.h |

### gpio_set_pupd

| Function Name | gpio_set_pupd |
| --- | --- |
| Syntax | GPIO_Status_t gpio_set_pupd(GPIO_TypeDef *port, uint8_t pin, GPIO_PUPD_t pupd) |
| Parameters (in) | port — GPIO port pointer pin — 0–15 pupd — NONE / PULLUP / PULLDOWN |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | GPIO_Status_t — GPIO_OK or GPIO_ERROR |
| Description | Configures PUPDR[2n+1:2n]. Enables or disables internal pull resistors. GPIO_PUPD_PULLDOWN is supported in this version (v1.1.0), unlike v1.0.0. |
| Constraints | External resistors take precedence over internal ones. Internal pull resistors are ~40 kΩ typical. |
| Available via | gpio_driver.h |

### gpio_set_af

| Function Name | gpio_set_af |
| --- | --- |
| Syntax | GPIO_Status_t gpio_set_af(GPIO_TypeDef *port, uint8_t pin, GPIO_AF_t af) |
| Parameters (in) | port — GPIO port pointer pin — 0–15 af — GPIO_AF0…GPIO_AF15 |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | GPIO_Status_t — GPIO_OK or GPIO_ERROR (if af > 15) |
| Description | Writes the 4-bit AF field in AFR[0] (pins 0–7, AFRL) or AFR[1] (pins 8–15, AFRH). The pin must be in GPIO_MODE_AF via gpio_init() first. For PA8: gpio_set_af(GPIOA, 8, GPIO_AF1) maps to TIM1_CH1. |
| Constraints | AF mapping is device-specific. Refer to STM32F411 datasheet Table 9 for full AF assignments. |
| Available via | gpio_driver.h |

### gpio_write

| Function Name | gpio_write |
| --- | --- |
| Syntax | void gpio_write(GPIO_TypeDef *port, uint8_t pin, uint8_t value) |
| Parameters (in) | port — GPIO port pointer pin — 0–15 value — 1 = drive HIGH, 0 = drive LOW |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | void |
| Description | Drives output pin atomically via BSRR. No read-modify-write on ODR — eliminates race conditions in concurrent contexts. value != 0 → BSRR[pin]=1 (set); value == 0 → BSRR[pin+16]=1 (reset). |
| Constraints | Pin must be configured as OUTPUT. No NULL/range check performed (void function). |
| Available via | gpio_driver.h |

### gpio_toggle

| Function Name | gpio_toggle |
| --- | --- |
| Syntax | void gpio_toggle(GPIO_TypeDef *port, uint8_t pin) |
| Parameters (in) | port — GPIO port pointer pin — 0–15 |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | void |
| Description | Inverts current pin output state via XOR on ODR. Used by led_toggle() in the LED blink loop. |
| Constraints | Non-atomic (read-modify-write on ODR). Do not use in concurrent contexts without mutual exclusion. |
| Available via | gpio_driver.h |

### gpio_read

| Function Name | gpio_read |
| --- | --- |
| Syntax | uint8_t gpio_read(GPIO_TypeDef *port, uint8_t pin) |
| Parameters (in) | port — GPIO port pointer pin — 0–15 |
| Parameters (inout) | None |
| Parameters (out) | None |
| Return value | uint8_t — 1 if pin is HIGH, 0 if LOW |
| Description | Reads current pin state from IDR register: (IDR >> pin) & 0x1U. |
| Constraints | Reads IDR regardless of pin mode. For output pins in push-pull mode, IDR reflects ODR state. Open-drain may differ. |
| Available via | gpio_driver.h |

## 4.4. Internal Function Definitions

### gpio_enable_clock (private)

| Function Name | gpio_enable_clock |
| --- | --- |
| Syntax | static void gpio_enable_clock(GPIO_TypeDef *port) |
| Parameters (in) | port — GPIOA…GPIOE pointer |
| Return value | void |
| Description | Sets the corresponding GPIOxEN bit in RCC->AHB1ENR using CMSIS named bit masks (RCC_AHB1ENR_GPIOAEN, etc.). Called automatically at the start of gpio_init(). |
| Constraints | Only handles GPIOA–GPIOE. Extend for GPIOH if needed. |

# 5. Future Enhancements

• Add GPIOH support in gpio_enable_clock() for NUCLEO boards that use it.

• Implement gpio_lock() using the LCKR register to protect critical pin configurations.

• Add gpio_set_exti() to configure EXTI interrupt lines for edge-triggered input events.

• Add input validation to gpio_write(), gpio_toggle(), and gpio_read() (currently void functions with no check).

• Support GPIO_SPEED_VHIGH documentation note for EMI considerations.